# Chapter 4 — Results and Discussion

## 4.1 Research Question 1

RQ1. How can the mathematical models and constraint validation logic for the furniture-specific 3D Bin Packing Problem with Sequential Loading Constraints (3DBPP-SLC) be formulated to strictly integrate orientation restrictions, multi-stop delivery LIFO constraints, and non-overlapping truck boundary limits?

This section answers Research Question 1 by presenting the complete mathematical formulation that drives the FLOW-3D solver pipeline, together with the independent constraint validation logic that enforces the same contract at runtime. The formulation is organized into seven constraint blocks. Blocks A through E reproduce the formal 3DBPP-SLC statement of thesis Section 3.5.2.1; Blocks F and G are implementation extensions required to eliminate two physically nonsensical optima that the A–E model admits (floating items and items stacked on fragile cargo). A final payload inequality is added as a single linear constraint on the packing binaries. All constraint families are mirrored in backend/core/validator.py as post-solve checks so that constraint satisfaction is verified independently of the path (ILP or FFD) taken by the hybrid engine.

### 4.1.1 Decision Variables and Domains

The model treats every item i in the input manifest as a candidate for inclusion in the loading plan of a truck with internal envelope W × L × H (millimetres). The decision variables follow the naming convention mandated by thesis Section 3.5.2.1 and are summarized below.

| Symbol | Domain | Meaning |
| --- | --- | --- |
| b_i | {0, 1} | 1 iff item i is packed into the truck |
| x_i, y_i, z_i | ℤ ≥ 0 | Position (mm) of the front-left-bottom corner of item i |
| o_{i,k} | {0, 1} | 1 iff item i takes orientation k ∈ {0, …, 5} |
| s_{ij,k} | {0, 1} | Separation indicator on plane k ∈ {1, …, 6} for the unordered pair {i, j} |
| floor_i | {0, 1} | 1 iff item i rests on the truck floor (Extension F) |
| u_{i,j} | {0, 1} | 1 iff item j is the unique direct supporter of item i (Extension F) |

The effective dimensions w_eff_i, l_eff_i, h_eff_i of item i are determined by the active orientation k through a fixed permutation map applied to the input tuple (w_i, l_i, h_i). Once orientation is fixed, all subsequent geometric constraints operate on the effective dimensions. A schematic of the variable geometry is shown in Figure 4.1, and the orientation enumeration is reproduced in Table 4.1.

### 4.1.2 Block A — Objective

The objective maximizes truck volumetric utilization, defined as the fraction of the internal truck volume occupied by packed items:

maximize    V_util = (1 / (W · L · H)) · Σ_i v_i · b_i                (A.1)

where v_i = w_i · l_i · h_i is the volume of item i in cubic millimetres. Because v_i is orientation-invariant, the objective is linear in the packing binaries alone and does not interact with the orientation variables. V_util is reported in the PackingPlan contract as a float in [0, 1] (CLAUDE.md, "PackingPlan contract").

### 4.1.3 Block B — Non-Overlap (Big-M Disjunction)

For every unordered pair of distinct items {i, j}, at least one separating plane must hold whenever both items are packed. Six binary indicators s_{ij,k} are introduced — one per face direction — and each is associated with a Big-M inequality that becomes vacuous when the corresponding indicator is zero:

x_i + w_eff_i ≤ x_j + M · (1 − s_{ij,1})                              (B.1)
x_j + w_eff_j ≤ x_i + M · (1 − s_{ij,2})                              (B.2)
y_i + l_eff_i ≤ y_j + M · (1 − s_{ij,3})                              (B.3)
y_j + l_eff_j ≤ y_i + M · (1 − s_{ij,4})                              (B.4)
z_i + h_eff_i ≤ z_j + M · (1 − s_{ij,5})                              (B.5)
z_j + h_eff_j ≤ z_i + M · (1 − s_{ij,6})                              (B.6)

Σ_{k = 1..6} s_{ij,k} ≥ b_i + b_j − 1                                 (B.7)

Inequality (B.7) forces at least one of the six planes to be active whenever both items are packed (b_i = b_j = 1), and leaves the separation indicators unconstrained otherwise. The constant M is chosen as max(W, L, H) so that any inactive plane is rendered trivially feasible. The six possible separation cases are illustrated in Figure 4.2.

### 4.1.4 Block C — Boundary

Each packed item must lie entirely within the truck envelope. The inequalities are gated by b_i so that unpacked items are exempt:

x_i + w_eff_i ≤ W · b_i                                               (C.1)
y_i + l_eff_i ≤ L · b_i                                               (C.2)
z_i + h_eff_i ≤ H · b_i                                               (C.3)

Combined with the integrality of the position variables, (C.1)–(C.3) guarantee that every packed item occupies an axis-aligned sub-box of the truck interior.

### 4.1.5 Block D — Orientation and Variable Domains

Each item is assigned exactly one orientation when packed and no orientation when unpacked:

Σ_{k = 0..5} o_{i,k} = b_i                                            (D.1)

The orientation index determines the effective dimensions through a fixed permutation table (Table 4.1). Rigid items carry a side_up flag in the input manifest that restricts the admissible orientation set. In the implementation, o_{i,k} is fixed to zero for every k outside the admissible set before the solver begins. This preserves linearity and lets Gurobi presolve eliminate the affected variables (backend/solver/ilp_solver.py).

### 4.1.6 Block E — Route-Sequenced LIFO

The truck loading axis is the Y-axis: y = 0 is the truck rear (the deepest point reached during loading), and y = L is the loading door. The LIFO requirement states that items destined for later stops must be loaded deeper, so that they sit behind earlier-stop items and are not blocked at unloading. For every ordered pair (i, j) of packed items with strictly differing stop identifiers,

stop_i > stop_j   ⟹   y_i + l_eff_i ≤ y_j                             (E.1)

Constraint (E.1) is added as a hard linear inequality for every such pair at model-build time; pairs sharing a stop are exempt, and pairs in the reverse stop order are covered by the symmetric instance of (E.1) with i and j swapped. The geometric interpretation of the LIFO axis is shown in Figure 4.3.

### 4.1.7 Block F — Vertical Support (Implementation Extension)

The A–E model admits floating items because Blocks B and C never require contact with a supporting surface. Extension F closes this gap using a single-supporter disjunction adapted from Bortfeldt and Mack (2007).

(F.1) Unique support. Every packed item is supported either by the floor or by exactly one other packed item:

floor_i + Σ_{j ≠ i} u_{i,j} = b_i                ∀ i                  (F.1)

(F.2) Floor lock. Floor-supported items have z-coordinate zero:

z_i ≤ H · (1 − floor_i)                          ∀ i                  (F.2)

(F.3) Supporter must be packed.

u_{i,j} ≤ b_j                                    ∀ i, j, i ≠ j        (F.3)

(F.4) Vertical contact. When j supports i, the bottom face of i coincides with the top face of j:

z_i − z_j − h_eff_j ≤ H · (1 − u_{i,j})          ∀ i, j, i ≠ j        (F.4a)
z_j + h_eff_j − z_i ≤ H · (1 − u_{i,j})          ∀ i, j, i ≠ j        (F.4b)

(F.5) Footprint containment. When j supports i, the xy footprint of i is fully contained within that of j:

x_j − x_i ≤ W · (1 − u_{i,j})                                       (F.5a)
(x_i + w_eff_i) − (x_j + w_eff_j) ≤ W · (1 − u_{i,j})               (F.5b)
y_j − y_i ≤ L · (1 − u_{i,j})                                       (F.5c)
(y_i + l_eff_i) − (y_j + l_eff_j) ≤ L · (1 − u_{i,j})               (F.5d)

The single-supporter approximation rules out items that physically span two adjacent supporters, but it keeps the formulation at O(n²) binaries and O(n²) constraints, which is essential to remain within the ILP regime (manifest size n ≤ SOLVER_THRESHOLD). Bortfeldt and Mack (2007) report that this approximation recovers near-optimal densities in practice for the 3DBPP. A cycle-freedom argument follows from (F.4): u_{i,j} = 1 forces `z_i = z_j + h_eff_j

z_jwheneverh_eff_j ≥ 1, so a reciprocal u_{j,i} = 1would implyz_j > z_i`, a contradiction. The support tree therefore contains no cycles and roots at the floor.

### 4.1.8 Block G — Fragile No-Stacking (Implementation Extension)

The FurnitureItem.fragile field on the data contract (backend/api/models.py:39) reads literally "Fragile — solver must not stack other items on top of this one." The constraint is implemented as an equality fixing on the supporter binary:

u_{i,j} = 0   ∀ i, j with j fragile                                   (G.1)

Combined with the unique-support equality (F.1), constraint (G.1) forces every item that would otherwise rest on j to choose the floor or some non-fragile supporter. If no such alternative exists, the affected item is left unpacked and surfaces in PackingPlan.unplaced_items. (G.1) is strictly tightening: it removes only solutions that violate the fragile contract and does not eliminate any physically valid packing. Note that (G.1) does not forbid a fragile item from being supported by something else — a fragile mirror may legally rest on top of a non-fragile crate — nor does it forbid items at the same height as a fragile item with no xy overlap. The constraint reserves only the vertical column directly above each fragile item.

### 4.1.9 Payload Constraint

The truck has a maximum payload mass payload_kg. A single linear inequality in the packing binaries enforces this limit:

Σ_i weight_kg_i · b_i ≤ payload_kg                                    (P.1)

Although (P.1) lies outside the formal 3.5.2.1 A–E statement, it is implemented in _weight() of backend/solver/ilp_solver.py and mirrored in validate_weight() of backend/core/validator.py. It introduces no new variables and is absorbed cleanly by Gurobi presolve.

### 4.1.10 Combined Model

The full integer linear program solved by ILPSolver for manifests of size n ≤ SOLVER_THRESHOLD is:

maximize    V_util = (1 / (W · L · H)) · Σ_i v_i · b_i               [A]

subject to  (B.1)–(B.7)   non-overlap (six Big-M planes)             [B]
            (C.1)–(C.3)   boundary containment                       [C]
            (D.1)         orientation assignment                     [D]
            (E.1)         route-sequenced LIFO                       [E]
            (F.1)–(F.5)   vertical single-supporter                  [F]
            (G.1)         fragile no-stacking                        [G]
            (P.1)         payload                                    [Pay]

            b_i, o_{i,k}, s_{ij,k}, floor_i, u_{i,j} ∈ {0, 1}
            x_i, y_i, z_i ∈ ℤ ≥ 0

For manifests larger than SOLVER_THRESHOLD, the same constraint contract is honoured by the FFDSolver heuristic (backend/solver/ffd_solver.py) through a deterministic Route-Sequential First-Fit Decreasing procedure: items are sorted by descending volume within increasing stop order, and each item is placed at the first feasible candidate position that satisfies all seven constraint families. The hybrid dispatch is described in Figure 4.4, and the per-operation complexities are reproduced in Table 4.2.

### 4.1.11 Constraint Validation Logic

The constraint validator (backend/core/validator.py::ConstraintValidator) is an independent post-solve component that re-verifies every constraint family against the returned PackingPlan. It is invoked unconditionally by the AbstractSolver template method after either solver path completes, so its verdict is independent of the path taken. This separation is enforced by the module rule that solver/ never imports from api/ or main.py, and that the validator lives in core/ (CLAUDE.md, "Module separation rules"). The mapping from validator methods to constraint blocks is:

| Validator method | Enforces | Predicate |
| --- | --- | --- |
| validate_boundary | Block C | For every placed item, 0 ≤ x, y, z and x + w ≤ W, y + l ≤ L, z + h ≤ H |
| validate_no_overlap | Block B | For every pair of placed items, their axis-aligned boxes are disjoint |
| validate_lifo | Block E | For every pair (a, b) with stop_a > stop_b, y_a + l_a ≤ y_b |
| validate_orientation | Block D | orientation_index ∈ {0, …, 5} and admissible under side_up |
| validate_no_stack_on_fragile | Block G | For every fragile placed item b and every other placed item a, if z_a ≥ z_b + h_b then the xy footprints of a and b are disjoint |
| validate_weight | Payload (P.1) | Σ weight_kg of placed items ≤ payload_kg |

The validate_no_stack_on_fragile predicate is strictly stronger than the support-level constraint (G.1): it inspects raw placement geometry rather than supporter binaries, so it catches any regression in either solver where a stacked-on-fragile placement could arise from a path that bypasses the support variables (e.g., a future bug in the FFD fragile gate). The validator's first_failing_check method returns a labelled diagnostic ("fragile_stacking", "lifo", "overlap", etc.) which is wrapped in PlanValidationError and surfaced to the frontend dashboard. The full validator-to-constraint mapping is also shown in Figure 4.5.

The architectural separation between the solver and the validator serves two purposes. First, it provides a runtime guarantee that the constraint contract holds for the FFD path, which is greedy and not globally optimal; without the validator, an FFD bug could produce a plan that violates LIFO or fragile no-stacking without detection. Second, it makes the constraint contract auditable: the validator file alone is sufficient to verify what the system promises, without the reader needing to follow the full Gurobi model build in ilp_solver.py.

### 4.1.12 Strict Integration of the Three RQ1 Requirements

Research Question 1 asks specifically how the three named requirement families — orientation restrictions, multi-stop LIFO, and non-overlapping truck boundary — are strictly integrated into a single mathematical model rather than enforced as independent post-processing filters. The integration is achieved through three design decisions, each visible in the formulation above.

(1) Shared effective dimensions tie orientation to every geometric constraint. The variables w_eff_i, l_eff_i, h_eff_i are functions of the orientation indicator o_{i,k} (Block D), and they appear in every spatial inequality — non-overlap (B), boundary (C), LIFO (E), vertical contact (F.4), and footprint containment (F.5). This means orientation is not a separate pre-processing step that fixes dimensions before the geometry is solved; instead, the solver jointly chooses orientation and position, and an invalid orientation choice for a rigid item is ruled out at the variable level by zeroing the inadmissible o_{i,k} entries.

(2) LIFO is a hard constraint of the same algebraic class as non-overlap. Constraint (E.1) is a linear inequality on the position variables and the effective length, structurally identical to a non-overlap plane in the y-direction. It is not enforced post-solve, nor as a soft penalty in the objective; the solver cannot return a plan that violates it. This is what distinguishes 3DBPP-SLC from plain 3DBPP, and what makes the formulation route-aware in the sense of the project title.

(3) Boundary, non-overlap, and LIFO share the packing binary gate. All three constraint families are scaled by b_i (boundary, C.1–C.3) or are conditioned on b_i + b_j − 1 (non-overlap, B.7) or on the implicit packed status of both endpoints (LIFO, E.1, which is trivially satisfied if either item is unpacked because the position of an unpacked item is unconstrained by Blocks B and C). This gating is what allows the model to partially pack a manifest when full inclusion is infeasible, returning the rejected items in unplaced_items rather than declaring the entire instance infeasible.

Together, these three design decisions produce a single ILP whose optimal solution is, by construction, simultaneously orientation- admissible, LIFO-compliant, and bounded by the truck envelope — and whose constraint contract is enforced again at runtime by the independent validator. The combined model is the formal answer to RQ1.

### 4.1.13 Summary of Findings for RQ1

The 3DBPP-SLC for furniture logistics can be formulated as a single integer linear program of O(n²) binaries and O(n²) constraints in the manifest size n. The formulation strictly integrates the three RQ1 requirements as follows:

- Orientation restrictions are encoded in Block D through selector variables o_{i,k} whose admissible set is restricted by the side_up flag, and which determine the effective dimensions used by every other geometric constraint.

- Multi-stop LIFO constraints are encoded in Block E as hard linear inequalities on the y-axis position variables, structurally parallel to the non-overlap planes and gated by stop identifier ordering.

- Non-overlapping truck boundary limits are encoded jointly in Blocks B (Big-M disjunctive separation, six planes) and C (envelope containment scaled by b_i).

Two implementation extensions (F, G) and one payload inequality augment the formal A–E statement to eliminate physically nonsensical optima and to honour the fragile and payload_kg fields of the data contract. The independent ConstraintValidator re-verifies every block against the returned PackingPlan, guaranteeing that the constraint contract holds regardless of which solver path (ILP or FFD) produced the plan.

A reference implementation of the formulation lives in backend/solver/ilp_solver.py; the parity heuristic in backend/solver/ffd_solver.py; and the post-solve validator in backend/core/validator.py. Empirical performance of the formulation against the configured SOLVER_THRESHOLD is presented in Section 4.2, which addresses Research Question 2.